Items:

- Low Noise Narrow Line-width less than 1kHz Laser ITU 34=1550.12nm

- Low Noise Narrow Line-width less than 1kHz Laser ITU 38=1546.92nm

- Low Noise Narrow Line-width less than 1kHz Laser ITU 42=1543.72nm

- Low Noise Narrow Line-width less than 1kHz Laser ITU 46=1540.56nm

Specification for Low Noise, narrow line-width laser sources

- Type of Laser: Low Phase noise, Narrow linewidth laser source with necessary accessories.

- Centre wavelength of the DFB laser emission:

  - ITU ch. 34 = 1550.12nm

  - ITU ch. 38 = 1546.92nm

  - ITU ch. 42 = 1543.72 nm

  - ITU ch. 46 = 1540.56 nm

  - A max deviation of ±0.05nm in center wavelength is only acceptable for a fixed operating temperature of 25°C

- Laser Emission: CW, inherently single frequency, with single polarization

- Power Output: ≥ 10mW

- Line Width: ≤ 1kHz

- Frequency Noise:

  - ≤ 100Hz/rt.Hz @ 1kHz to 10kHz

  - ≤ 30Hz/rt.Hz @ 10kHz to 1MHz

  - ≤ 10Hz/rt.Hz @ >1MHz

- Relative Intensity Noise (RIN): ≤−110dBc/Hz @ 1kHz

- RIN Relaxation Frequency: > 100 kHz

- Optical S/N: ≥ 50dB

- Longitudinal Lasing Modes: Single Mode

- Polarization Extinction Ratio: ≥ 20dB

- Pigtail fiber PM fiber with at least 1m length, laser dominant polarization aligned to slow axis

- Fiber connector at ends FC/APC male connector with the slow axis aligned to key

- Package option Well packaged against shock, vibration etc.

- Control Interface Preferably USB/ RS232/ RS485 or equivalent

- Accessories Provide all the essential accessories like Power supply adaptor (for 230V, 50Hz), cables, GUI controller etc., if they are required for the operation mentioned.

- Test Report and Certificate of Conformance: Test report and Certificate of Conformance to be delivered for each item

- Warranty: Two years